### BUG list – 17/06/2025 to 23/06/2025

### Bug List Index

17-06-2025 -06-2025
 1. ADC data validity issue
 2. FIFO write limitation issue 
 3. FIFO read overwrite issue 
 4. FIFO data skipping during write

18-06-2025
 5. Invalid data stored despite timeout
 6. ADC data capture issue (extension of point 1)
 7. Analog delay misbehavior (extension of point 5)
 8. Analog delay cycle mismatch
 9. Timeout not triggered after analog_done (extension of point 7)

20-06-2025
 10. Timeout counter starts before start_analog
 11. Analog vs control clock ratio constraint
 12. Missed start_slot due to clock domain timing

23-06-2025
 13. Timeout counter not initialized properly
 14. Minimum delay constraint between start and mid slots
 15. FIFO skips values after 7th write
 16. Timeout counter inactive during mid slot
 17. Multiple start_analog triggers before analog_done
 18. Timeout counter inactive after start slot
 19. wr_ptr not resetting without global reset

17-06-2025

1. ADC Data Validity Note - SOLVED

In the design, ADC data is valid only when the EOC (End of Conversion) signal is high.
 This EOC signal remains high for 5 clock cycles, during which the data output from the ADC is stable.

In the waveform below, you can see that the ADC provides valid data during the EOC high window.
 However, due to incorrect capture timing, the data was not sampled properly during this window, and as a result, it was not stored into memory.

2.  FIFO Write Issue Observation - SOLVED

Even though ADC data remains valid for a long time (since EOC is high for 5 cycles), when I attempt to write 128 values, the FIFO only accepts 76 entries — it doesn’t take more than that.

Because of this:

- The low storage warning is not triggered.

- The FIFO does not reach full, so we are unable to test if it correctly prevents overwriting when full.

Note:
 This behavior is due to Point 4 — only half the data is being written because of skipped writes (e.g., data_2 is missed while data_1 and data_3 are captured).

However, when 256 values are written continuously:

- The FIFO does get filled, and

- The low storage warning is correctly raised.

This confirms that the FIFO works properly, but write logic needs to be checked for data skipping in smaller transfers.

3.  FIFO Read Issue Observation – SOLVED (overwrite was allowed)

Even though the ADC data remains stable more than the entire duration of EOC, and only half the expected data is stored in the FIFO, an issue occurs during reading.

When reading from the FIFO:

- Data is not being removed (popped) after each read. --> overwrite is allowed

- As a result, the same value appears repeatedly, and the FIFO content doesn’t update as expected.

4.  FIFO Write Skipping Issue - SOLVED

When writing data into the FIFO, I observe that some values are getting skipped.

For example:

- I write data_1, data_2, data_3 in consecutive cycles.

- But inside the FIFO, only data_1 and data_3 are stored — data_2 is missing.

With the analog timeout

18-06-2025

### 5. Invalid Data Stored Despite Timeout

Even though a timeout occurred — meaning the data was marked as invalid — the system still stored this invalid data into the FIFO.

Here only the timeout reference is wrong the functionality is correct

19-06-2025

6. Extension of Point 1 – ADC Data Capture Issue – SOLVED (top level design change wasn't informed )

This issue is an extension of Point 1.

Earlier, it was assumed that ADC data would remain stable for a longer time, so capturing it directly was working.
 Now, the code has been modified to latch the ADC data internally into a register (e.g., adc_reg) before using it.

However, this new approach is not working correctly:

- The actual ADC data is valid and correct.

- But the internal register (adc_reg) shows XX values

7. Extension of Point 5 – Analog Delay Misbehavior - SOLVED

This issue extends Point 5.

I was testing a case where the analog delay is set differently for start_slot and mid_slot.
 The goal was to verify that if either delay exceeds the maximum allowed analog delay, the corresponding data should not be written.

However, while testing this:

- I noticed that the analog start detection is triggered only by start_slot.

- The mid_slot does not generate its own analog start — instead, it reuses or copies the analog data associated with start_slot.

- As a result, the ADC receives incorrect data during the mid slot, defeating the purpose of having separate analog delays.

8. Analog Delay Handling – SOLVED

The max_analog_delay value is used for delay counting in both the analog module and the controller module.
 However:

- In the analog module, the delay is calculated per ana_clk cycle.

- In the controller module, the same delay is interpreted per ctrl_clk cycle.

This creates a mismatch when:

- ana_clk is faster than ctrl_clk
 → The analog module believes it is delaying for a valid duration less than or equal to max_analog_delay,
 → But from the controller's perspective, the effective delay exceeds max_analog_delay.

9. Extension of Point 7 – Timeout Behavior - SOLVED

This issue is an extension of Point 7.

From the waveform, we can see that:

- The timeout condition occurs only at the beginning.

- After that, no further timeout is triggered, even though it should be — because the system immediately detects analog_done.

20-06-2025

10. Extension of Point 9 – Timeout Counter Error - SOLVED

This is an extension of Point 9.

Another issue with the timeout logic is that the timeout counter starts counting even before start_analog is asserted.

As a result:

- The timeout may expire prematurely, before the analog delay even begins.

- This leads to false timeout detections, even when the analog module is functioning correctly.

11. Design Constraint – Maximum Ratio Between ana_clk and ctrl_clk

Due to the system’s functional behavior, ana_clk cannot be more than 1.33× slower than ctrl_clk.
 This constraint comes from the way the timing is handled between start_analog, analog_done, and their respective detection points.

Timing Breakdown:

- From start_analog to analog_start_detected → 5 ana_clk cycles

- After that, analog delay = ana_delay cycles (in ana_clk)

- From analog_done to analog_done_detected → 2 ctrl_clk cycles

So the total time for the analog transaction is:

(5 + ana_delay) * ana_clk_period + 2 * ctrl_clk_period

To meet the constraint of maximum allowed analog delay (in controller time), we require:

(5 + ana_delay) * ana_clk_period + 2 * ctrl_clk_period < max_analog_delay * ctrl_clk_period

Finding the Limit:

Give ana_delay lower limit to find upper limit of ana_clk_period , max_analog_delay = 10

(5 + 1) * ana_clk_period + 2 * ctrl_clk_period < 10 * ctrl_clk_period

6 * ana_clk_period < 8 * ctrl_clk_period

ana_clk_period < (8 / 6) * ctrl_clk_period

ana_clk_period < 1.33 * ctrl_clk_period

12. Clock Domain Timing Issue – Missed start_slot

In this case, you can observe that the counter runs on the ctrl_clk domain, while the start_slot signal is generated based on the rising edge of the timing_clk.

If timing_clk is slower than ctrl_clk, the following issue can occur:

- The counter reaches specific values (e.g., 0, 1, etc.) before the rising edge of timing_clk.

23-06-2025

13. Incorrect Timeout Counter Initialization

The timeout counter does not start correctly, causing the actual analog delay to exceed the user-specified max_analog_delay.

14. Design Constraint: Minimum Delay Between Start and Mid Slot

To ensure correct operation and prevent premature slot detection due to synchronization latency, the following design constraint must be enforced:

The delay between start_slot and mid_slot must be greater than 36 controller clock cycles.

Rationale:

- The start_slot_detected signal is generated after a synchronization delay of approximately 4 controller clock cycles.

- The programmable delay counter begins after this detection.

- To guarantee that the next slot does not occur before the delay period completes, a minimum of 36 controller clock cycles is required between start_slot and mid_slot, including synchronization overhead.

15. FIFO Skipping Values After 7th Write – solved

In this case, the FIFO begins to skip values starting from the 8th write, even though the ADC data is valid and the programmable delays are within the specified range.

To demonstrate this, a timeout trace has been plotted alongside the captured data. The trace confirms that each data point falls within a valid programmable delay window, and the ADC is triggered correctly.

This indicates that the issue is not with analog timing or timeout logic

Debug: Checker Mismatch Due to Timing Clock Change

Following recent changes to the timing clock period, a few test checkers did not correctly account for the updated clock behavior. As a result:

- The computed delta between start_slot and mid_slot became smaller than the intended valid range.

- This led to incorrect behavior in the test, including false failures or unexpected results.

The issue was not in the design itself but in the testbench assumptions and setup.
 It has now been resolved by updating the relevant test cases to align with the new timing configuration, ensuring that all slot intervals fall within valid limits.

16. Timeout Counter Not Active During Mid Slot - SOLVED

Due to the current implementation, the timeout counter is only active during the START slot and not during the MID slot. This behavior occurs because:

- The counter is reset as soon as analog_done is detected, which typically happens at the end of the START state.

- The counter only starts again in the START state of the next capture cycle.

- As a result, during the MID state, the timeout counter is not running, and no timeout detection occurs for that phase.

This causes the design to miss timeout conditions during the MID slot, leading to incorrect acceptance of delayed or invalid data. The timeout logic needs to be extended or duplicated for the MID slot to ensure proper coverage and correctness.

Debug: Designer didn't realize it was a feature of their FSM

17. Multiple start_analog Triggers Before analog_done Causes False Timeout - SOLVED

In the current behavior, multiple start_analog signals are being generated before a single analog_done is detected. This leads to a critical issue:

- Each new start_analog resets or restarts the timeout counter.

- If analog_done does not arrive in time relative to the latest start_analog, the system incorrectly flags a timeout, even though the data may still be valid from the initial trigger.

This results in false timeout detections, which can cause valid ADC data to be incorrectly discarded or skipped.

Debug: A design constraint was violated

18. Timeout Counter Not Active After Start Slot - SOLVED

In the highlighted case, the start_slot signal has arrived, but the timeout counter is not running as expected. This leads to missed timeout detection.

Debug: A design constraint was violated

19. wr_ptr Does Not Reset to Zero Without Global Reset

The write pointer (wr_ptr) logic in the design is structured such that it does not return to 0 unless a full system reset is applied.

As a result:

- The condition wr_ptr_bin < rd_ptr_bin is never satisfied during normal operation.

- This affects code coverage, as any logic depending on wr_ptr wrapping or resetting is never exercised